5. IP Addressing Plan

10.28.0.0/15 is the designated address block. Each VLAN is assigned a /24 to enable straightforward Packet Tracer configuration and significant spare capacity for future users.

| VLAN | Department | Network | Subnet Mask | Default Gateway | Usable Hosts |
| --- | --- | --- | --- | --- | --- |
| 10 | Management | 10.28.10.0/24 | 255.255.255.0 | 10.28.10.1 | 254 |
| 20 | Finance | 10.28.20.0/24 | 255.255.255.0 | 10.28.20.1 | 254 |
| 30 | Reception | 10.28.30.0/24 | 255.255.255.0 | 10.28.30.1 | 254 |
| 40 | Operations | 10.28.40.0/24 | 255.255.255.0 | 10.28.40.1 | 254 |
| 50 | Guest Services | 10.28.50.0/24 | 255.255.255.0 | 10.28.50.1 | 254 |
| 99 | Network Management | 10.28.99.0/24 | 255.255.255.0 | 10.28.99.1 | 254 |

5.1 Infrastructure Addressing Convention

1 — Default gateway

2–10 Network infrastructure

11+ — Servers, printers and reserved devices

50+ — User devices

DHCP can be used for ordinary client PCs later, while infrastructure devices should use predictable addresses